To do for mechanical (October 26 2025)

### Step 9: Define Structural Loads

Load Case 1: Vertical Load (weight of electronics)

- Click "Structural Constraints" tab

- Click "Fixed"

  - Select: Bottom mounting surface (chassis attachment)

  - This is where the bracket is bolted to rover chassis

- Click "Force"

  - Select: Top mounting surfaces (all 4 circles)

  - Direction: -Z (downward)

  - Magnitude: 30 N (3kg × 10 = 30N, simulating 3kg electronics bay)

Load Case 2: Lateral Load (rover bumps/turns)

- Click "Add Load Case"

- Click "Fixed" → Select bottom mounting surface again

- Click "Force"

  - Select: Top mounting surfaces

  - Direction: -X (sideways)

  - Magnitude: 15 N (lateral acceleration)

Load Case 3: Impact Load (landing after jump)

- Click "Add Load Case"

- Click "Fixed" → Select bottom mounting surface

- Click "Force"

  - Select: Top mounting surfaces

  - Direction: -Z

  - Magnitude: 90 N (3× weight, simulating 3G impact)

## Part 5: Configure Study Settings

### Step 10: Material and Manufacturing

Material:

- Click "Material" tab

- Select: PLA (3D Printing) or PETG

  - If not listed, use: "Plastic - Generic"

  - Density: 1.24 g/cm³

  - Tensile Strength: 50 MPa

Manufacturing:

- Click "Manufacturing" tab

- Method: Additive Manufacturing

- Overhang Angle: 45 degrees (standard for 3D printing without supports)

- Minimum Thickness: 2.5mm (ensures printability)

Objectives:

- Click "Objectives" tab

- Select: Minimize Mass (primary goal)

- Safety Factor: 2.0 (ensures 2× stronger than needed)

### Step 11: Generate

- Click "Generate" (top right)

- Resolution: Start with "Medium" (faster, ~30 min)

  - Later you can re-run with "High" for final design

- Click "Generate"