# Rover Communication & Control Software Overview

### System Architecture

The rover uses two communication channels:

- Wi-Fi for high-speed control, video streaming, and nearby operation.

- LoRa for long-range telemetry and backup command link.

The system has three major computing units:

- Jetson Nano – main processor and vision/computation unit.

- Teensy MCU – low-level motor and sensor controller.

- Base Station (Laptop) – operator interface and monitoring system.

## 1. Teensy Firmware

Role:

- Acts as the rover’s real-time controller (“muscles”).

- Executes movement commands and collects sensor data.

Responsibilities:

- Read sensors: motor encoders, IMU, GPS, and battery voltage.

- Drive motors via PWM.

- Parse and execute incoming movement commands.

- Communicate over two UART channels:

  - To Jetson Nano: high-speed control link.

  - To LoRa module: telemetry and long-range commands.

- Send periodic telemetry packets to both links.

## 2. Jetson Nano Software

Role:

- Main “brain” of the rover.

- Handles vision, autonomy, and short-range Wi-Fi communication.

Responsibilities:

- Receive movement commands from the base station (via Wi-Fi).

- Forward control instructions to the Teensy.

- Collect sensor feedback from the Teensy.

- Optionally stream camera video to the base station (via WebRTC, RTSP, or MJPEG).

- Host a local control interface or API endpoint for manual or autonomous driving modes.

Interfaces:

- Serial connection to the Teensy.

- Network connection (Wi-Fi) to the operator console.

## 3. LoRa Communication Layer

Role:

- Provides reliable long-range, low-bandwidth link.

Rover-side responsibilities:

- Transmit compact telemetry data (GPS, voltage, system status).

- Receive backup or emergency commands (STOP, RETURN, etc.).

- Operate independently of Wi-Fi availability.

Base-side responsibilities:

- Receive and display rover telemetry.

- Allow the operator to send basic commands even when Wi-Fi is out of range.

## 4. Base Station Software

Role:

- Operator control interface and mission dashboard.

Responsibilities:

- Display live video feed from the Jetson Nano.

- Show telemetry (position, battery, status) from LoRa.

- Provide control inputs (joystick, keyboard, or on-screen buttons).

- Send high-frequency commands over Wi-Fi; low-frequency or fail-safe commands over LoRa.

## 5. Communication Protocol

| Link | Direction | Typical Data | Notes |
| --- | --- | --- | --- |
| Wi-Fi | Bidirectional | Control commands, video, high-rate telemetry | Short-range, low latency |
| LoRa | Bidirectional | GPS, status, slow control | Long-range, high reliability |
| UART (Jetson ↔ Teensy) | Bidirectional | Motor commands, sensor feedback | Local wired link |
| UART (Teensy ↔ LoRa) | Bidirectional | Telemetry + backup commands | Text-based packets |

All packets should follow a simple key-value format (human-readable for debugging).

## 6. Recommended Development Sequence

- Teensy motor and sensor testing – verify local control.

- Jetson-Teensy link – send commands, receive feedback.

- LoRa telemetry – test long-range data transmission.

- Wi-Fi control interface – implement real-time manual control.

- Integrate both links – Wi-Fi as primary, LoRa as fallback.

- Video streaming integration – finalize operator interface.

## 7. Summary of Roles

| Component | Language | Primary Functions |
| --- | --- | --- |
| Teensy MCU | C/C++ | Motor control, sensor input, serial communication |
| Jetson Nano | Python or C++ | High-level logic, vision, Wi-Fi networking |
| LoRa Modules | Firmware / UART | Telemetry and backup control |
| Base Station | Python, Web, or GUI | Operator interface, video display, mission control |

Result:
 A robust, dual-link control architecture where Wi-Fi provides real-time operation and video, while LoRa ensures persistent telemetry and backup control under all conditions.

LAPTOP

├─→ LoRa USB Dongle ──(915MHz)──→ LoRa Tracker → Teensy 4.1 (Motors)

└─→ WiFi ──(2.4/5GHz)──────────→ Jetson Nano (Camera/AI)

↓ USB Serial

Teensy 4.1 (coordination)